Robert Herriot (editor)

Sun Microsystems

Sylvan Butler

Hewlett-Packard

Paul Moore
Microsoft.

Randy Turner
Sharp Labs

July 14, 1997

Internet Printing Protocol/1.0: Protocol Specification

draft-ietf-ipp-protocol-00.txt

Status of this Memo

This document is an Internet-Draft.  Internet-Drafts are working documents of the Internet Engineering Task Force (IETF), its areas, and its working groups.  Note that other groups may also distribute working documents as Internet-Drafts.

Internet-Drafts are draft documents valid for a maximum of six months and may be updated, replaced, or obsoleted by other documents at any time.  It is inappropriate to use Internet-Drafts as reference material or to cite them other than as "work in progress".

To learn the current status of any Internet-Draft, please check the "1id-abstracts.txt" listing contained in the Internet-Drafts Shadow Directories on ftp.is.co.za (Africa), nic.nordu.net (Europe), munnari.oz.au (Pacific Rim), ds.internic.net (US East Coast), or ftp.isi.edu (US West Coast).

Abstract

This document is one of a set of documents, which together describe all aspects of a new Internet Printing Protocol (IPP).  IPP is an application level protocol that can be used for distributed printing using Internet tools and technology.  The protocol is heavily influenced by the printing model introduced in the Document Printing Application (ISO/IEC 10175 DPA) standard.  Although DPA specifies both end user and administrative features, IPP version 1.0 is focused only on end user functionality.

The full set of IPP documents includes:

Internet Printing Protocol: Requirements

Internet Printing Protocol/1.0: Model and Semantics

Internet Printing Protocol/1.0: Security

Internet Printing Protocol/1.0: Protocol Specification

Internet Printing Protocol/1.0: Directory Schema

The requirements document takes a broad look at distributed printing functionality, and it enumerates real-life scenarios that help to clarify the features that need to be included in a printing protocol for the Internet.  It identifies requirements for three types of users: end users, operators, and administrators.  The requirements document calls out a subset of end user requirements that MUST be satisfied in the first version of IPP.  Operator and administrator requirements are out of scope for v1.0. The model and semantics document describes a simplified model with abstract objects, their attributes, and their operations. The model introduces a Printer object and a Job object.  The Job object supports multiple documents per job.  The security document covers potential threats and proposed counters to those threats.  The protocol specification is formal document which incorporates the ideas in all the other documents into a concrete mapping using clearly defined data representations and transport protocol mappings that real implementers can use to develop interoperable client and server side components. Finally, the directory schema document shows a generic schema for directory service entries that represent instances of IPP Printers.

This document is the "Internet Printing Protocol/1.0: Protocol Specification" document.

Table of Contents

1. Introduction

2. Conformance Terminology

3. Encoding of  the Operation Layer

3.1 Picture of the Encoding

3.2 Syntax of Encoding

3.3 Version

3.4 Mapping of Operations

3.5 Mapping of Status-code

3.6 Tags

3.7 Name-Lengths

3.8 Mapping of Parameter Names

3.9 Value Lengths

3.10 Mapping of Attribute and Parameter Values

3.11 Data

4. Encoding of Transport Layer

4.1 General Headers

4.2 Request  Headers

4.3 Response Headers

4.4 Entity  Headers

5. Security Considerations

6. Appendix A: Protocol Examples

6.1 Print-Job Request

6.2 Print-Job Response (successful)

6.3 Print-Job Response (failure)

6.4 Print-URI Request

6.5 Create-Job Request

6.6 Get-Jobs Request

6.7 Get-Jobs Response

7. Appendix B: Requirements without HTTP/1.1 as a Transport Layer

7.1 Additional Parameter-group for HTTP header information

7.2 Chunking of Data

7.3 Revised Picture for the Operation Layer

7.4 Revised Syntax for the Operation Layer

8. Appendix C: Mapping of Operation Parameters to the their Position in the Encoding

9. References

10. Author's Address

11. Other Participants:

# Introduction

This document contains the rules for encoding IPP operations and describes two layers: the transport layer and the operation layer.

The transport layer consists of an  HTTP/1.1 request or response. RFC 2068 [27] describes HTTP/1.1. This document specifies the HTTP headers that an IPP implementation supports.

The operation layer consists of  a message body in an HTTP request or response.  The document "Internet Printing Protocol/1.0: Model and Semantics" [21] defines the semantics of such a message body and the supported values. This document specifies the encoding of an IPP operation. The aforementioned document [21] is henceforth referred to as the “IPP model document”

# Conformance Terminology

The key words "MUST", "MUST NOT", "REQUIRED", "SHALL", "SHALL NOT", "SHOULD", "SHOULD NOT", "RECOMMENDED", "MAY", and  "OPTIONAL" in this document are to be interpreted as described in RFC 2119 [25].

# Encoding of  the Operation Layer

The operation layer SHALL contain a single operation request or operation response.

The encoding consists of two primitive types.  integers and characters, on which all other data types are built. Every character in this encoding SHALL be a member of the UCS-2 coded character set and SHALL be encoded using UTF-8  Every integer in this encoding SHALL be encoded in binary as a signed integer with big-endian format (also known as “network order” and “most significant byte first”). The number of bytes for an integer SHALL be a power of 2, that is, 1, 2 and 4.

The following two sections present the operation layer in two ways

- informally through pictures and description

- formally through Augmented Backus-Naur Form (ABNF), as specified by draft-ietf-drums-abnf-02.txt [29]

## Picture of the Encoding

The encoding for an operation request or response consists of:

- -----------------------------------------------

|                    version                  |   2 bytes  - required

-----------------------------------------------

|operation (request) or status-code (response)|   2 bytes  - required

-----------------------------------------------------------

|                 parameter-tag               |   1 byte  |

-----------------------------------------------           |- optional

|               parameter-sequence            |   m bytes |

-----------------------------------------------------------

|                 attribute-tag               |   1 byte  |

-----------------------------------------------           |- 0 or more

|               attribute-sequence            |   n bytes |

-----------------------------------------------------------

|                  data-tag                   |   1 byte   - required

-----------------------------------------------

|                     data                    |   q bytes  - optional

-----------------------------------------------

The parameter-tag and parameter-sequence may be omitted if the operation has no parameters. The attribute-tag and attribute-sequence may be omitted if the operation has no attributes or it may be replicated for an operation that contains attributes for multiple objects. The data is omitted from some operations, but the data-tag is present even when the data is omitted. Note, the parameter-tag, attribute-tag and data-tag are called ‘delimiter-tags’.

A parameter-sequence consists of  a sequence of zero or more compound-parameters.

-----------------------------------------------

|               compound-parameter            |   r bytes - 0 or more

-----------------------------------------------

An attributes-sequence consists of zero or more compound-attributes.

-----------------------------------------------

|              compound-attribute             |   s bytes - 0 or more

-----------------------------------------------

A compound-parameter consists of a parameter with a single value optionally followed by zero or more additional values. A compound-attribute consists an attribute with a single value followed by zero or more additional values.

Each parameter or attribute consists of:

-----------------------------------------------

|                   value-tag                 |   1 byte

-----------------------------------------------

|               name-length  (value is u)     |   2 bytes

-----------------------------------------------

|                     name                    |   u bytes

-----------------------------------------------

|              value-length  (value is v)     |   2 bytes

-----------------------------------------------

|                     value                   |   v bytes

-----------------------------------------------

An additional value consists of:

-----------------------------------------------------------

|                   value-tag                 |   1 byte  |

-----------------------------------------------           |

|            name-length  (value is 0x0000)   |   2 bytes |

-----------------------------------------------           |- 0 or more

|              value-length (value is w)      |   2 bytes |

-----------------------------------------------           |

|                     value                   |   w bytes |

-----------------------------------------------------------

Note: an additional value is like a parameter or attribute whose name-length is 0.

From the standpoint of a parsing loop, the encoding consists of:

-----------------------------------------------

|                    version                  |   2 bytes  - required

-----------------------------------------------

|operation (request) or status-code (response)|   2 bytes  - required

-----------------------------------------------------------

|        tag (delimiter-tag or value-tag)     |   1 byte  |

-----------------------------------------------           |- 0 or more

|      empty or rest of parameter/attribute   |   x bytes |

-----------------------------------------------------------

|                   data-tag                  |   2 bytes  - required

-----------------------------------------------

|                     data                    |   y bytes  - optional

-----------------------------------------------

The value of the tag determines whether the bytes following the tag are:

- parameters

- attributes

- data

- the remainder of a single parameter or attribute where the tag specifies the type of the value.

## Syntax of Encoding

The syntax below is ABNF except ‘strings of literals’ SHALL be case sensitive. For example ‘a’ means lower case  ‘a’ and not upper case ‘A’.   In addition, two-byte binary signed integer fields are represented as ‘%x’ values which show their range of values.

ipp-message = ipp-request / ipp-response

ipp-request = version operation [parameter-tag parameter-sequence ]
         *(attribute-tag  attribute-sequence) data-tag  data

ipp-response = version status-code [parameter-tag parameter-sequence ] 
         *(attribute-tag attribute-sequence)  data-tag  data

version = major-version minor-version

major-version = SIGNED-BYTE  ; initially %d1

minor-version = SIGNED-BYTE  ; initially %d0

operation = SIGNED-SHORT  ; mapping from model defined below

status-code = SIGNED-SHORT     ; mapping from model defined below

parameter-sequence =   *compound-parameter

attribute-sequence = *compound-attribute

compound-parameter = parameter *additional-values

compound-attribute = attribute *additional-values

parameter = value-tag name-length name value-length value

attribute = value-tag name-length name value-length value

additional-values = value-tag zero-name-length value-length value

name-length = SIGNED-SHORT    ; number of octets of ‘name’

name = LALPHA *( LALPHA / DIGIT / “-” / “_” )

value-length = SIGNED-SHORT  ; number of octets of ‘value’

value = OCTET-STRING

data = OCTET-STRING

zero-name-length = %x00 %x00            ; name-length of 0

parameter-tag =  %x01  ; tag of 1

attribute-tag =  %x02    ; tag of 2

data-tag = %x03                ; tag of 3

value-tag = %x10..%xFF

SIGNED-BYTE = %x00..%xFF

SIGNED-SHORT = %x00..%xFF %x00..%xFF

DIGIT = "0".."9"

LALPHA = "a".."z"

BYTE = %x00..%xFF

OCTET-STRING = *BYTE

The syntax allows a parameter-tag to be present when the parameter-sequence that follows is empty. The same is true for the attribute-tag and the attribute-sequence that follows. The syntax is defined this way to allow for the response of Get-Jobs where no attributes are returned for some job-objects.  Although it is RECOMMENDED that the sender not send a parameter-tag if there are no parameters and not send an attribute-tag if there are no attributes (except in the Get-Jobs response just mentioned), the receiver MUST be able to decode such syntax.

Version

The version SHALL consist of a major and minor version, each of which SHALL be represented by a one byte signed integer. The protocol described in this document SHALL have a major version of 1 (0x01) and a minor version of  0 (0x00).  The ABNF for these two bytes SHALL be %x01.00.

## Mapping of Operations

The following SHALL be the mapping of operations names to integer values which are encoded as two byte binary signed integers. The operations are defined in the IPP model document  The table below includes a range of values for future extensions to the protocol and a separate range for private extensions.  It is RECOMMENDED that the private extension values be used for temporary experimental implementations and not for deployed products.

| Encoding (Hex) | Operation |
| --- | --- |
| 0 | reserved (not used) |
| 1 | Get-Operations |
| 2 | Print-Job |
| 3 | Print-URI |
| 4 | Validate-Job |
| 5 | Create-Job |
| 6 | Send-Document |
| 7 | Send-URI |
| 8 | Cancel-Job |
| 9 | Get-Attributes |
| A | Get-Jobs |
| B-3FFF | reserved for future operations |
| 4000-FFFF | reserved for private extensions |

## Mapping of Status-code

The following SHALL be the mapping of status-code names to integer values which are encoded as two byte binary signed integers. The status-code names are defined in the IPP model document.

If an IPP status-code is returned, then the HTTP Status-Code MUST be 200 (OK). With any other HTTP Status-Code value, the HTTP response SHALL NOT contain an IPP message-body, and thus no IPP status-code is returned.

Note: the integer encodings below were chosen to be similar to corresponding Status-Code values in HTTP. The IPP client and server errors have the same relative offset to their base as corresponding HTTP errors, but the IPP base is a multiple of 0x100 whereas the HTTP base is a multiple of 100. Despite this similarity,  the Status-Code returned at the HTTP level will always be different except in the case where ‘OK’ is returned at both levels, 200 (OK) in HTTP and 0 (successful-OK) in IPP.

Note: some status-code values, such as client-error-unauthorized, may  be returned at the transport (HTTP) level rather than the operation level.

ISSUE: as implementations proceed, we will learn what error code need to be supported at the IPP level.

| Encoding (Hex) | Status-Code Name |
| --- | --- |
| 0 | successful-OK |
| 0x400 | client-error-bad-request |
| 0x401 | client-error-unauthorized |
| 0x403 | client-error-forbidden |
| 0x404 | client-error-not-found |
| 0x405 | client-error-method-not-allowed |
| 0x408 | client-error-timeout |
| 0x40A | client-error-gone |
| 0x40D | client-error-request-entity-too-large |
| 0x40E | client-error-request-URI-too-long |
| 0x40F | client-error-unsupported-document-format |
| 0x410 | client-error-attribute-not-supported |
| 0x500 | server-error-internal-server-error |
| 0x501 | server-error-operation-not-implemented |
| 0x503 | server-error-service-unavailable |
| 0x504 | server-error-timeout |
| 0x505 | server-error-version-not-supported |
| 0x506 | server-error-printer-error |
| 0x507 | server-error-temporary-error |

## Tags

There are two kinds of tags:

- delimiter tags: delimit major sections of the protocol, namely parameters, attributes and data

- value tags: specify the type of each parameter or attribute value

The following table specifies the values for the delimiter tags:

| Tag Value (Hex) | Delimiter |
| --- | --- |
| 0x00 | reserved |
| 0x01 | parameter-tag |
| 0x02 | attribute-tag |
| 0x03 | data-tag |
| 0x04-0x0F | reserved for future delimiters |

The remaining tables show values for the value-tag which is the first octet of a parameter or attribute. The value-tag specifies the type of the value of the parameter or attribute. The value of the value-tag of a parameter or attribute SHALL either be a type value specified in the model document or an “out-of-band” value, such as “unsupported” or “default”. If  the value of value-tag for a attribute or parameter is not “out-of-band” and differs from the value type specified by the model document, then a server receiving such a request MAY reject it, and  a client receiving such a response MAY ignore the attribute or parameter.

The following table specifies the “out-of-band” values for the value-tag.

| Tag Value (Hex) | Meaning |
| --- | --- |
| 0x10 | unsupported |
| 0x11 | default |
| 0x12-0x1F | reserved for future “out-of-band” values. |

The “unsupported” value SHALL be used in the attribute-sequence of an error response for those attributes which the server does not support. The “default” value is reserved for future use of setting value back to their default value.

The following table specifies the integer values for the value-tag

| Tag Value (Hex) | Meaning |
| --- | --- |
| 0x20 | reserved |
| 0x21 | integer (up to 4 bytes) |
| 0x22 | boolean |
| 0x23 | seconds |
| 0x24 | milliseconds |
| 0x25 | enum |
| 0x26-0x3F | reserved for future integer types |

NOTE: 0x20 is reserved for “generic integer” if should ever be needed.

The following table specifies the character-string values for the value-tag

| Tag Value (Hex) | Meaning |
| --- | --- |
| 0x40 | reserved |
| 0x41 | text |
| 0x42 | name |
| 0x43 | filename |
| 0x44 | keyword |
| 0x45 | uri |
| 0x46 | uriScheme |
| 0x47 | dateTime |
| 0x48-0x7F | reserved for future character string types |

NOTE: 0x40 is reserved for “generic character-string” if should ever be needed.

The values 0x80-0xFF are reserved for future types.

## Name-Lengths

The name-length field SHALL consist of a two byte binary signed integer in big endian order. This field SHALL specify the number of octets in the name field which follows the name-length field, excluding the two bytes of the ‘name-length’ field.

If a name-length field has a value of zero, the following name field SHALL be empty, and the following value SHALL be treated as an additional value for the preceding parameter. Within a parameter-sequence, if two parameters have the same name, the first occurrence SHALL be ignored. Within an attribute-sequence, if two attributes have the same name, the first occurrence SHALL be ignored. The zero-length name is the only mechanism for multi-valued parameters and attributes.

Mapping of Parameter Names

Some parameters are encoded in a special position.  These parameters are:

- “request-URI”: The request-URI of each operation in the IPP model document SHALL be specified outside of  the operation layer as the request-URI on the Request-Line at the HTTP level, and SHALL not be specified within the operation layer.

- “document-content”: The parameter named “document-content” in the IPP model document SHALL become the “data” in the operation layer.

- “status-code”: The parameter named “status-code” in the IPP model document SHALL become the “status-code” field in the operation layer response.

ISSUE: Should the request-URI be outside the operation layer?

The remaining parameters are encoded in the parameter-sequence or the attribute-sequence.  The parameter-sequence is for actual operation parameters and the attribute-sequence is for object attributes. Of the parameters defined in the IPP model document, some represent an actual operation parameters and others represent a collection of object attributes.

A parameter in the IPP model document SHALL represent a collection of object attributes if its name contains the word “attributes”  immediately preceded by a space.; otherwise it SHALL represent an actual operation parameter. Note, some actual operation parameters contain the word “attributes” preceded by a hyphen (“-”); they are not a collection of attributes.

If a parameter in IPP model document represents an actual operation parameter and is not in a special position, it SHALL be encoded in the parameter-sequence using the text name of the parameter specified in the IPP model document.

If a parameter in IPP model document represents a collection of object attributes, the attributes SHALL be encoded in the attribute-sequence using the text names of the attributes specified in the IPP model document. The IPP model document specifies the members of such attribute collections, but  does not require that all members of a collection be present in an operation.

If an operation contain attributes from exactly one object, there SHALL be exactly one attribute-sequence. If an operation contains attributes from more than one object (e.g. Get-Jobs response), the attributes from each object SHALL be in a separate attribute-sequence, such that the attributes from the ith object are in the ith attribute-sequence.

See  Section 7 “Appendix C: Mapping of Operation Parameters to the their Position in the Encoding” for table showing the application of the rules above.

## Value Lengths

Each parameter value SHALL be preceded by a two byte binary signed integer in big endian order which SHALL specify the number of octets in the value which follows this length, exclusive of the two bytes specifying the length.

For any of the types represented by binary signed integers, the sender MAY encode the value in fewer than the maximum 4 bytes, but the number of bytes for the encoding MUST be a power of two, i.e. 1, 2 or 4, and representation MUST be as a signed integer in the fewer bytes.

ISSUE: allowing 4 byte integers to be transmitted in 1 or 2 bytes, at client discretion, may be more trouble than the saved bytes are worth.  Do we really want this?

For any of the types represented by character-strings, the sender MUST encode the value with all the characters of the string and without any padding characters.

If a value-tag contains an “out-of-band” value, such as “unsupported”, the value-length SHALL be 0 and the value empty — the value has no meaning when the value-tag has an “out-of-band” value. If a server or client receives an operationwith a nonzero value-length in this case, it SHALL ignore the value field.

Mapping of Attribute and Parameter Values

The following SHALL be the mapping of attribute and parameter values to their IPP encoding in the value field. The syntax types are defined in the IPP model document.

| Syntax of Attribute Value | Encoding |
| --- | --- |
| text | an octet string where each character is a member of the UCS-2 coded character set and is encoded using UTF-8. The text is encoded in “network byte order” with the first character in the text (according to reading order) being the first character in the encoding. |
| name | same as text |
| fileName | same as text |
| keyword | same as text. Allowed text values are defined in the IPP model document |
| uri | same as text |
| uriScheme | same as text |
| boolean | one binary octet where 0x00 is ‘false’ and 0x01 is ‘true’ |
| integer | number of octets is a power of 2 (i.e. 1, 2 or 4). These octets represent a binary signed integer in big endian order (also known as “network byte order” and MSB first). |
| enum | same as integer. Allowed integer values are defined in the IPP model document |
| dateTime | same as text. Syntax of data and time is defined by ISO 8601 ISSUE: should ISO 8601 be called out in the IPP model document? |
| seconds | same as integer |
| milliseconds | same as integer |
| 1setOf X | encoding according to the rules for a parameter with more than more value. Each value X is encoded according to the rules for encoding its type. |
| rangeOf X | same 1setOf X where the number of values is 2. |

The type of the value in the model document determines the encoding in the value and the value of the value-tag.

There is sometimes a need for a parameter to have some special ‘out-of-band’ values.  Such value are represented by empty values with special negative lengths as specified by the table below.

If a response contains a parameter of “unsupported attribute”, the value of “unsupported” shall be used to denote that the Printer does not support the attribute.

ISSUE: The above sentence belongs in the IPP model document.

## Data

The data part SHALL include any data required by the operation

Encoding of Transport Layer

HTTP/1.1 shall be the transport layer for this protocol.

The operation layer has been designed with the assumption that the transport layer contains the following information:

- the URI of the target job or printer operation

- the total length of the data in the operation layer, either as a single length or as a sequence of chunks each with a length.

- the client’s language, the character-set and the transport encoding.

Each HTTP operation shall use the POST method where the request-URI is the object target of the operation, and where the “Content-Type” of the message-body in each request and response shall be “application/ipp”. The message-body shall contain the operation layer and shall have the syntax described in section 2 “Conformance Terminology”.

A client implementation SHALL adhere to the rules for a client described in RFC 2068. A server implementation SHALL adhere the rules for an origin server described in RFC 2068

ISSUE: Should the URI of the operation be in the operation layer? Should the URI in the POST be a server/printer always with the target object as a parameter within the message-body?

In the following sections, there are a tables of all HTTP headers which describe their use in an IPP client or server.  The following is an explanation of each column in these tables.

- the “header” column contains the name of a header

- the “request/client” column indicates whether a client sends the header.

- the “request/server” column indicates whether a server supports the header when received.

- the “response/server” column indicates whether a server sends the header.

- the “response /client” column indicates whether a client supports the header when received.

- the “values and conditions” column specifies the allowed header values and the conditions for the header to be present in a request/response.

The table for “request headers” does not have columns for responses, and the table for “response headers” does not have columns for requests.

The following is an explanation of the values in the “request/client” and “response/server” columns.

- man: (mandatory) the client or server MUST send the header,

- c-man: (conditionally mandatory)  the client or server MUST send the header when the condition described in the “values and conditions” column is met,

- opt: (optional) the client or server MAY send the header

- not: (not needed) the client or server SHOULD NOT send the header. It is not relevant to an IPP implementation.

The following is an explanation of the values in the “response/client” and “request/server” columns.

- man: (mandatory) the client or server MUST support the header,

- opt: (optional)  the client or server MAY support the header

- not: (not needed) the client or server SHOULD NOT support the header. It is not relevant to an IPP implementation.

## General Headers

The following is a table for the general headers.

ISSUE: an HTTP expert should review these tables for accuracy.

| General-Header | Request |  | Response |  | Values and Conditions |
| --- | --- | --- | --- | --- | --- |
|  | Client | Server | Server | Client |  |
| Cache-Control | man | not | man | not | “no-cache” only |
| Connection | c-man | man | c-man | man | “close” only. Both client and server SHOULD keep a connection for the duration of a sequence of operations. The client and server MUST include this header for the last operation in such a sequence. |
| Date | opt | opt | man | opt | per RFC 1123 [9] |
| Pragma` | man | not | man | not | “no-cache” only |
| Transfer-Encoding | c-man | man | c-man | man | “chunked” only . Header MUST be present if Content-Length is absent. |
| Upgrade | not | not | not | not |  |
| Via | not | not | not | not |  |

## Request  Headers

The following is a table for the request headers.

| Request-Header | Client |  | Server | Request Values and Conditions |
| --- | --- | --- | --- | --- |
| Accept | opt | man |  | “application/ipp” only. This value is the default if the client omits it |
| Accept-Charset | opt | man |  | per IANA Character Set registry. ISSUE: is this useful for IPP? |
| Accept-Encoding | opt | man |  | empty and per RFC 2068 [27] and IANA registry for content-codings |
| Accept-Language | opt | man |  | see RFC 1766 [26]. A server SHOULD honor language requested by returning the values status-message, job-state-message and printer-state-reason in one of requested languages. |
| Authorization | c-man | man |  | per RFC 2068. A client MUST send this header when it receives a 401 “Unauthorized” response and does not receive a “Proxy-Authenticate” header. |
| From | not | not |  | per RFC 2068. Because RFC recommends sending this header only with the user’s approval, it is not very useful |
| Host | man | man |  | per RFC 2068 |
| If-Match | not | not |  |  |
| If-Modified-Since | not | not |  |  |
| If-None-Match | not | not |  |  |
| If-Range | not | not |  |  |
| If-Unmodified-Since | not | not |  |  |
| Max-Forwards | not | not |  |  |
| Proxy-Authorization | c-man | not |  | per RFC 2068. A client MUST send this header when it receives a 401 “Unauthorized” response and a “Proxy-Authenticate” header. |
| Range | not | not |  |  |
| Referer | not | not |  |  |
| User-Agent | not | not |  |  |

## Response Headers

The following is a table for the request headers.

| Response-Header | Server | Client | Response Values and Conditions |
| --- | --- | --- | --- |
| Accept-Ranges | not | not |  |
| Age | not | not |  |
| Location | c-man | opt | per RFC 2068. When URI needs redirection. |
| Proxy-Authenticate | not | man | per RFC 2068 |
| Public | opt | opt | per RFC 2068 |
| Retry-After | opt | opt | per RFC 2068 |
| Server | not | not |  |
| Vary | not | not |  |
| Warning | opt | opt | per RFC 2068 |
| WWW-Authenticate | c-man | man | per RFC 2068. When a server needs to authenticate a client. |

## Entity  Headers

The following is a table for the entity headers.

| Entity-Header | Request |  | Response |  | Values and Conditions |
| --- | --- | --- | --- | --- | --- |
|  | Client | Server | Server | Client |  |
| Allow | not | not | not | not |  |
| Content-Base | not | not | not | not |  |
| Content-Encoding | opt | man | man | man | per RFC 2068 and IANA registry for content codings. |
| Content-Language | opt | man | man | man | see RFC 1766 [26]. |
| Content-Length | c-man | man | c-man | man | the length of the message-body per RFC 2068. Header MUST be present if Transfer-Encoding is absent.. |
| Content-Location | not | not | not | not |  |
| Content-MD5 | opt | opt | opt | opt | per RFC 2068 |
| Content-Range | not | not | not | not |  |
| Content-Type | man | man | man | man | “application/ipp” only |
| ETag | not | not | not | not |  |
| Expires | not | not | not | not |  |
| Last-Modified | not | not | not | not |  |

# Security Considerations

When utilizing HTTP 1.1 as a transport of IPP, the security considerations outlined in RFC 2068 apply.  Specifically, IPP servers can generate a 401 "Unauthorized" response code to request client authentication and IPP clients should correctly respond with the proper "Authorization" header.  Both Basic Authentication (RFC 2068) and Digest Authentication (RFC 2069) flavors of authentication should be supported.  The server chooses which type(s) of authentication to accept.  Digest Authentication is a more secure method, and is always preferred to Basic Authentication.

For secure communication (privacy in particular), IPP should be run using a secure communications channel.  Both Transport Layer Security - TLS (draft-ietf-tls-protocol-03) and IPSec (RFC 1825) provide necessary features for security.  It is possible to combine the two techniques, HTTP 1.1 client authentication (either basic or digest) with secure communications channel (either TLS or IPSec).  Together the two are more secure than client authentication and they perform user authentication.

Complete discussion of IPP security considerations can be found in the IPP  Security document

Appendix A: Protocol Examples

## Print-Job Request

The following is an example of a Print-Job request with job-name, copies, and sides specified.

| Octets | Symbolic Value | Protocol field |
| --- | --- | --- |
| 0x0100 | 1.0 | version |
| 0x0002 | PrintJob | operation |
| 0x02 | start attributes | attribute tag |
| 0x42 | name type | value-tag |
| 0x0008 |  | name-length |
| job-name | job-name | name |
| 0x0006 |  | value-length |
| foobar | foobar | value |
| 0x21 | integer type | value-tag |
| 0x0005 |  | name-length |
| copies | copies | name |
| 0x0001 |  | value-length |
| 0x14 | 20 | value |
| 0x44 | keyword type | value-tag |
| 0x0005 |  | name-length |
| copies | sides | name |
| 0x0001 |  | value-length |
| two-sided-long-edge | two-sided-long-edge | value |
| 0x03 | start-data | data-tag |
| %!PS... | <PostScript> | data |

## Print-Job Response (successful)

Here is an example of a Print-Job response which is successful:

| Octets | Symbolic Value | Protocol field |  |
| --- | --- | --- | --- |
| 0x0100 | 1.0 | version |  |
| 0x0000 | OK (successful) | status-code |  |
| 0x01 | start parameters | parameter tag |  |
| 0x41 | text type | value-tag |  |
| 0x000E |  | name-length |  |
| status-message | status-message | name |  |
| 0x0002 |  | value-length |  |
| OK | OK | value |  |
| 0x45 | uri type | value-tag |  |
| 0x0008 |  | name-length |  |
| job-uri | job-uri | name |  |
| 0x000E |  | value-length |  |
| http://foo/123 | http://foo/123 | value |  |
| 0x02 | start attributes | attribute tag |  |
| 0x25 | name type | value-tag |  |
| 0x0008 |  | name-length |  |
| job-state | job-state | name |  |
| 0x0001 |  | value-length |  |
| 0x03 | pending | value |  |
| 0x03 | start-data | data-tag |  |

## Print-Job Response (failure)

Here is an example of a Print-Job response which fails because the printer does not support sides and because the value 20 for copies is not supported:

| Octets |  | Symbolic Value | Protocol field |  |  |
| --- | --- | --- | --- | --- | --- |
| 0x0100 |  | 1.0 | version |  |  |
| 0x0400 |  | client-error-bad-request | status-code |  |  |
| 0x01 |  | start parameters | parameter tag |  |  |
| 0x41 |  | text type | value-tag |  |  |
| 0x000E |  |  | name-length |  |  |
| status-message |  | status-message | name |  |  |
| 0x000D |  |  | value-length |  |  |
| bad-request |  | bad-request | value |  |  |
| 0x02 |  | start attributes | attribute tag |  |  |
| 0x21 | integer type |  | value-tag |  |  |
| 0x0005 |  |  | name-length |  |  |
| copies | copies |  | name |  |  |
| 0x0001 |  |  | value-length |  |  |
| 0x14 | 20 |  | value |  |  |
| 0x10 | unsupported (type) |  | value-tag |  |  |
| 0x0005 |  |  | name-length |  |  |
| copies | sides |  | name |  |  |
| 0x0000 |  |  | value-length |  |  |
| 0x03 |  | start-data | data-tag |  |  |

## Print-URI Request

The following is an example of Print-URI request with copies and job-name parameters.

| Octets | Symbolic Value | Protocol field |
| --- | --- | --- |
| 0x0100 | 1.0 | version |
| 0x0003 | Print-URI | operation |
| 0x01 | start-parameters | parameter tag |
| 0x45 | uri type | value-tag |
| 0x000A |  | name-length |
| document-uri | document-uri | name |
| 0x11 |  | value-length |
| ftp://foo.com/foo | ftp://foo.com/foo | value |
| 0x02 | start-attributes | attribute tag |
| 0x42 | name type | value-tag |
| 0x0008 |  | name-length |
| job-name | job-name | name |
| 0x0006 |  | value-length |
| foobar | foobar | value |
| 0x21 | integer type | value-tag |
| 0x0005 |  | name-length |
| copies | copies | name |
| 0x0001 |  | value-length |
| 0x01 | 1 | value |
| 0x03 | start-data | data-tag |
| %!PS... | <PostScript> | data |

## Create-Job Request

The following is an example of Create-Job request with no parameters and no attributes

| Octets | Symbolic Value | Protocol field |
| --- | --- | --- |
| 0x0100 | 1.0 | version |
| 0x0005 | Create-Job | operation |
| 0x03 | start-data | data-tag |

## Get-Jobs Request

The following is an example of Get-Jobs request with parameters but no attributes.

| Octets | Symbolic Value | Protocol field |
| --- | --- | --- |
| 0x0100 | 1.0 | version |
| 0x000A | Get-Jobs | operation |
| 0x01 | start-parameters | parameter-tag |
| 0x21 | integer type | value-tag |
| 0x0005 |  | name-length |
| limit | limit | name |
| 0x0001 |  | value-length |
| 0x32 | 50 | value |
| 0x44 | keyword type | value-tag |
| 0x0014 |  | name-length |
| requested-attributes | requested-attributes | name |
| 0x0007 |  | value-length |
| job-uri | job-uri | value |
| 0x44 | keyword type | value-tag |
| 0x0000 | additional value | name-length |
| 0x0008 |  | value-length |
| job-name | job-name | value |
| 0x03 | start-data | data-tag |

## Get-Jobs Response

The following is an of Get-Jobs response from previous request with 3 jobs. The Printer returns no information about the second job.

| Octets | Symbolic Value | Protocol field |
| --- | --- | --- |
| 0x0100 | 1.0 | version |
| 0x0000 | OK (successful) | status-code |
| 0x01 | start-parameters | parameter-tag |
| 0x41 | text type | value-tag |
| 0x000E |  | name-length |
| status-message | status-message | name |
| 0x0002 |  | value-length |
| OK | OK | value |
| 0x02 | start-attributes (1st object) | attribute-tag |
| 0x45 | uri type | value-tag |
| 0x0007 |  | name-length |
| job-uri | job-uri | name |
| 0x000E |  | value-length |
| http://foo/123 | http://foo/123 | value |
| 0x42 | name type | value-tag |
| 0x0008 |  | name-length |
| job-name | job-name | name |
| 0x0003 |  | name-length |
| foo | foo | name |
| 0x02 | start-attributes (2nd object) | attribute-tag |
| 0x02 | start-attributes (3rd object) | attribute-tag |
| 0x45 | uri type | value-tag |
| 0x0007 |  | name-length |
| job-uri | job-uri | name |
| 0x000E |  | value-length |
| http://foo/124 | http://foo/124 | value |
| 0x42 | name type | value-tag |
| 0x0008 |  | name-length |
| job-name | job-name | name |
| 0x0003 |  | name-length |
| bar | bar | name |
| 0x03 | start-data | data-tag |

# Appendix B: Requirements without HTTP/1.1 as a Transport Layer

The operation layer defined above assumed certain services would be provided by the HTTP transport layer.  Without that layer, information, such as length, request-URI and Content-Encoding are absent.

This section specifies the modifications to the operation-layer encoding for raw TCP/IP. The following changes would have to made. All of these changes are upward compatible with the encoding defined in section 3 “Encoding of  the Operation Layer”.

## Additional Parameter-group for HTTP header information

There is an additional header-sequence which is like a parameter-sequence. The header-sequence SHALL appear as the  before  the parameter-sequence, and it SHALL contain the “request-URI” along with relevant HTTP header information, including those shown below. This header-sequence SHALL be preceded by a header-tag .

The following table shows the mapping of  HTTP headers to parameters in the operation layer.

| HTTP header or other concept | IPP header name | Syntax Type of header |
| --- | --- | --- |
| URI | request-URI | uri |
| Connection | Close-Connection | Boolean |
| Accept-Charset | Accept-Charset | keyword |
| Accept-Encoding | Accept-Encoding | keyword |
| Accept-Language | Accept-Language | keyword |
| Content-Encoding | Content-Encoding | keyword |
| Content-Language | Content-Language | keyword |
| charset parameter | Content-Charset | keyword |
| miscellaneous security | if needed at this level |  |

The first parameter in the header-sequence for a request SHALL be the attribute “request-URI” which is the target object of the operation.

Except for Content-Encoding, the headers  SHALL take effect at the beginning of the parameter-sequence and apply to the rest of the operation. If the header is Content-Encoding, then the encoding SHALL apply only to the ‘full-data’ or ‘data-segment’s as defined by the syntax below and the resulting decoded data SHALL have the syntax of all octets that follow a header-sequence. The syntax in a section below defines the syntax following a header-sequence to be:

[ parameter-tag parameter-sequence ] *(attribute-tag attribute-sequence) data

From a decoding point of view, if Content-Encoding is specified, the operation’s data is decoded using the algorithm specified by Content-Encoding.  The resulting octet stream is parsed as if it were the octets following a header-sequence.

NOTE: This rule for Content-Encoding allows a client or server to encode the parameter-sequence and attribute-sequences  or to transmit them un-encoded.

ISSUE: should the status-message be in the initial parameter group  instead of the second one for responses?

## Chunking of Data

There is an new delimiter tag called ‘chunked-data-tag’ which denotes that the following data is chunked. Chunked data consists of a sequence of data-segment-lengths and data-segments.  A data-segment-length of 0 denotes the end of the data.

The following is the informal picture of  chunked-data:

-----------------------------------------------------------

|             data-segment-length             |   2 bytes |

-----------------------------------------------           |- 0 or more

|                data-segment                 |   n bytes |

-----------------------------------------------------------

|             0x0000 (end-of-data)            |   2 bytes   - required

-----------------------------------------------

The syntax for the chunked-data using ABNF is:

chunked-data = *data-chunk END-OF-DATA

data-chunk = data-segment-length data-segment

data-segment-length = SIGNED-SHORT   ; number of octets of the data in binary

data-segment = OCTET-STRING

END-OF-DATA = %x00.00

A data-segment contains fragments of the data. When all the data-segments are concatenated together, they form the complete data.

## Revised Picture for the Operation Layer

The encoding for an operation request or response consists of:

-----------------------------------------------

|                    version                  |   2 bytes  - required

-----------------------------------------------

|operation (request) or status-code (response)|   2 bytes  - required

-----------------------------------------------------------

|                   header-tag                |   1 byte  |

-----------------------------------------------           |- optional

|                header-sequence              |   k bytes |

-----------------------------------------------------------

|                 parameter-tag               |   1 byte  |

-----------------------------------------------           |- optional

|               parameter-sequence            |   m bytes |

-----------------------------------------------------------

|                 attribute-tag               |   1 byte  |

-----------------------------------------------           |- 0 or more

|               attribute-sequence            |   n bytes |

-----------------------------------------------------------

|                  data-tag                   |   1 byte   - required

-----------------------------------------------

|                     data                    |   q bytes  - optional

-----------------------------------------------

The definition of all fields above is the same as defined in the main body of this document except

- ‘data’ which either has the syntax defined in the preceding section or the syntax described in the main body of this document.

- ‘header-tag’ which is new and defined like ‘parameter-tag’.

- ‘header-sequence’ which is new and defined like ‘parameter-sequence’.

## Revised Syntax for the Operation Layer

The following is the revised syntax for the operation layer.

ipp-message = ipp-request / ipp-response

ipp-request = version operation [ header-tag  header-sequence ]

[ parameter-tag parameter-sequence ] *(attribute-tag attribute-sequence) data

ipp-response = version status-code [ header-tag header-sequence ]

[ parameter-tag parameter-sequence ]  *(attribute-tag attribute-sequence) data

version = major-version minor-version

major-version = SIGNED-BYTE  ; initially %d1

minor-version = SIGNED-BYTE  ; initially %d0

operation = SIGNED-SHORT  ; mapping from model defined below

status-code = SIGNED-SHORT     ; mapping from model defined below

header-sequence = *compound-header

parameter-sequence =   *compound-parameter

attribute-sequence = *compound-attribute

compound-header = header *(additional-values)

compound-parameter = parameter *(additional-values)

compound-attribute = attribute *(additional -values)

header = value-tag name-length name value-length value

parameter = value-tag name-length name value-length value

attribute = value-tag name-length name value-length value

additional-values = value-tag zero-name-length value-length value

name-length = SIGNED-SHORT    ; number of octets of ‘name’

name = LALPHA *( LALPHA / DIGIT / “-” / “_” )

value-length = SIGNED-SHORT  ; number of octets of ‘value’

value = OCTET-STRING

data = (data-tag  full-data ) / (chunked-data-tag chunked-data )

full-data = OCTET-STRING

chunked-data = *data-chunk end-of-data

data-chunk = data-segment-length data-segment

data-segment-length = SIGNED-SHORT   ; number of octets of the data in binary

data-segment = OCTET-STRING

zero-name-length = %x00 %x00            ; name-length of 0

parameter-tag =  %x01           ; tag of 1

attribute-tag =  %x02             ; tag of 2

data-tag = %x03                         ; tag of 3

chunked-data-tag = %x04     ; tag of 4

header-tag = %x05                    ; tag of 5

value-tag = %x10..%xFF

end-of-data = %x00.00

SIGNED-BYTE = %x0..%xFF

SIGNED-SHORT = %x0..%xFF %x0..%xFF

DIGIT = "0".."9"

LALPHA = "a".."z"

BYTE = %x0..%xFF

OCTET-STRING = *BYTE

# Appendix C: Mapping of Operation Parameters to the their Position in the Encoding

The next three tables show the results of applying the rules above to the operations defined in the IPP model document. There is no information in these tables that cannot be derived from the rules presented in Section  “Mapping of Parameter Names”.

The following table shows the mapping of all IPP model document request parameters (except request-URI) to a parameter-sequence, attribute-sequence  or special position in the protocol.

| Operation | parameter-sequence | attribute-sequence | special position |
| --- | --- | --- | --- |
| Get-Operations |  |  |  |
| Print-Job |  | job-template attributes | document-content |
| Validate-Job or Create-Job |  | job-template attributes |  |
| Print-URI | document-uri | job-template attributes |  |
| Send-Document | last-document | document attributes | document-content |
| Send-URI | last-document document-uri | document attributes |  |
| Cancel -Job | message |  |  |
| Get-Attributes | document-format requested-attributes |  |  |
| Get-Jobs | limit requested-attributes |  |  |

The following table shows the mapping of all IPP model document response parameters to a parameter-sequence, attribute-sequence or special position in the protocol.

| Operation | parameter-sequence | attribute-sequence | special position |
| --- | --- | --- | --- |
| Get-Operations | status-message supported-operations |  | status-code |
| Print-Job, Print-URI or Create-Job | status-message job-uri | job-status attributes | status-code |
| Send-Document or Send-URI | status-message | job-status attributes | status-code |
| Validate-Job | status-message |  | status-code |
| Cancel-Job | status-message |  | status-code |
| Get-Attributes | status-message | requested attributes | status-code |
| Get-Jobs | status-message | requested attributes (see the Note below) | status-code |

Note for Get-Jobs: there is a separate attribute-sequence  containing requested-attributes for each job object in the response

The following table shows the mapping of all IPP model document error response parameters to a parameter-sequence, attribute-sequence or special position in the protocol. Those operations omitted don’t have special parameters for an error return.

| Operation | parameter-sequence | attribute-sequence | special position |
| --- | --- | --- | --- |
| Print-Job, Print-URI, Validate-Job, Create-Job, Send-Document or Send-URI | status-message | unsupported attributes | status-code |

# References

[1]	Smith, R., Wright, F., Hastings, T., Zilles, S., and Gyllenskog, J., "Printer MIB", RFC 1759, March 1995.

[2]	Berners-Lee, T, Fielding, R., and Nielsen, H., "Hypertext Transfer Protocol - HTTP/1.0", RFC 1945, August 1995.

[3]	Crocker, D., "Standard for the Format of ARPA Internet Text Messages", RFC 822, August 1982.

[4]	Postel, J., "Instructions to RFC Authors", RFC 1543, October 1993.

[5]	ISO/IEC 10175 Document Printing Application (DPA), June 1996.

[6]	Herriot, R. (editor), X/Open A Printing System Interoperability Specification (PSIS), August 1995.

[7]	Kirk, M. (editor), POSIX System Administration - Part 4: Printing Interfaces, POSIX 1387.4 D8, 1994.

[8]	Borenstein, N., and Freed, N., "MIME (Multi-purpose Internet Mail Extensions) Part One: Mechanism for Specifying and Describing the Format of Internet Message Bodies", RFC 1521, September, 1993.

[9]	Braden, S., "Requirements for Internet Hosts - Application and Support", RFC 1123, October, 1989,

[10]	McLaughlin, L. III, (editor), "Line Printer Daemon Protocol" RFC 1179, August 1990.

[11]	Berners-Lee, T., Masinter, L., McCahill, M. , "Uniform Resource Locators (URL)", RFC 1738, December, 1994.

[20]	Wright, F. D., “Internet Printing Protocol: Requirements”

[21]	Isaacson, S, deBry, R, Hasting, T, Herriot, R, Powell, P, “Internet Printing Protocol/1.0: Model and Semantics”

[22]	Internet Printing Protocol/1.0: Security

[23]	Herriot, R, Butler, S, Moore, P, Turner, R, “Internet Printing Protocol/1.0: Protocol Specification”  (This document)

[24]	Carter, K, Isaacson, S, “Internet Printing Protocol/1.0: Directory Schema”

[25]	S. Bradner, "Key words for use in RFCs to Indicate Requirement Levels", RFC 2119 , March 1997

[26]	H. Alvestrand, " Tags for the Identification of Languages", RFC 1766, March 1995.

[27]       R Fielding, et al, “Hypertext Transfer Protocol – HTTP/1.1” RFC 2068, January 1997

[28]       Marcus Kuhn, “International Standard Date and Time Notation”, ISO 8601, 
http://www.ft.uni-erlangen.de/~mskuhn/iso-time.html

[29]	D. Crocker et al., “Augmented BNF for Syntax Specifications: ABNF”, draft-ietf-drums-abnf-02.txt.

# Author's Address

Robert Herriot (editor)

Sun Microsystems Inc.

901 San Antonio.Road, MPK-17

Palo Alto, CA 94303

Phone: 415-786-8995

Fax:	 415-786-7077

Email: robert.herriot@eng.sun.com

Sylvan Butler

Hewlett-Packard

11311 Chinden Blvd.

Boise, ID 83714

Phone: 208-396-6000

Fax:	 208-396-3457

Email: sbutler@boi.hp.com

Paul Moore

Microsoft

One Microsoft Way

Redmond, WA 98053

Phone: 425-936-0908

Fax: 425-93MS-FAX

Email: paulmo@microsoft.com

Randy Turner

Sharp Laboratories

5750 NW Pacific Rim Blvd

Camas, WA 98607

Phone: 360-817-8456

Fax: : 360-817-8436

Email: rturner@sharplabs.com

IPP Mailing List:  ipp@pwg.org

IPP Mailing List Subscription:  ipp-request@pwg.org

IPP Web Page:  http://www.pwg.org/ipp/

# Other Participants:

Chuck Adams - Tektronix

Ron Bergman - Data Products

Keith Carter - IBM

Angelo Caruso - Xerox

Jeff Copeland - QMS

Roger Debry - IBM

Lee Farrell - Canon

Sue Gleeson - Digital

Charles Gordon - Osicom

Brian Grimshaw - Apple

Jerry Hadsell - IBM

Richard Hart - Digital

Tom Hastings - Xerox

Stephen Holmstead

Zhi-Hong Huang - Zenographics

Scott Isaacson - Novell

Rich Lomicka - Digital

David Kellerman - Northlake Software

Robert Kline - TrueSpectra

Dave Kuntz - Hewlett-Packard

Takami Kurono - Brother

Rich Landau - Digital

Greg LeClair - Epson

Harry Lewis - IBM

Tony Liao - Vivid Image

David Manchala - Xerox

Carl-Uno Manros - Xerox

Jay Martin - Underscore

Larry Masinter - Xerox

Bob Pentecost - Hewlett-Packard

Patrick Powell - SDSU

Jeff Rackowitz - Intermec

Xavier Riley - Xerox

Gary Roberts - Ricoh

Stuart Rowley - Kyocera

Richard Schneider - Epson

Shigern Ueda - Canon

Bob Von Andel - Allegro Software

William Wagner - Digital Products

Jasper Wong - Xionics

Don Wright - Lexmark

Rick Yardumian - Xerox

Lloyd Young - Lexmark

Peter Zehler - Xerox

Frank Zhao - Panasonic

Steve Zilles - Adobe